- FAA and KAZC permits (update 2020)

  - Open blank Excel Spreadsheet

    - Add columns for Northing, easting, LAT, Long, elevation coordinates from Mircostation for each signal pole.

    - Northing and easting coordinates only convert into LAT/LONG deg,min,sec (ex. Remove the degrees, minutes, and secs symbols, 38 12 12.123 -85 34 23.12 (add negative sign)).  You can use http://kgs.uky.edu/kgsweb/CoordConversionTool.asp to convert.  Check if coordinates are close enough to location by bringing up the map on this website.

    - Email excel spreadsheet to AirportZoning@ky.gov to see if we need to apply for a permit.

      - If the response is a no, you need to keep email for the records and complete the following:

        - Input all locations in to the FAA Notice criteria tool,  https://oeaaa.faa.gov/oeaaa/external/gisTools/gisAction.jsp?action=showNoNoticeRequiredToolForm

        - Add column to spreadsheet to note if the tool requires a FAA permit.

        - If the tool requires permit, go to d

          - If structure is shielded by a permanent structure contact Division of traffic operations to see if design services agrees to use FAA requirements under 77.9 e (1).

            - If yes go to step d (ii).

            - If no go to step d.

        - Does the map on the screen show a heliport?  FAA has jurisdiction over all private and public heliports.  KAZC only has jurisdiction over public heliports (Kentucky has no public heliports).  A heliport will look like a perfect circle on the map.  Usually the airport will have an eclipse and an approach block.

          - If yes you will need to file for a permit for all locations. Go to step e.

          - If no you will need to document that you do not see a heliport surface. Then you would need to document (with photos and added to file) if location fits within the 77.9 e 1 requirements to not file a permit:

(e) You do not need to file notice for construction or alteration of: (1) Any object that will be shielded by existing structures of a permanent and substantial nature or by natural terrain or topographic features of equal or greater height, and will be located in the congested area of a city, town, or settlement where the shielded structure will not adversely affect safety in air navigation; You are done.

        - You will need to get a login ID and fill out a permit for each location.  Make sure that you make KYTC the sponsor and add Jessica Goodwin as the contact for the Sponsor.

        - You may be required by the FAA to update or modify your information that has been turned in on the permit.

        - FAA also will require you to turn in a PART 2 submittal after the structure is installed.  If the structure is moved in the field, a new permit shall be submitted to the FAA.  You will need to terminate the existing location permit.

      - If the response is a yes, you will need to do the following;

        - Get and fill out the permit application (TC 55-2) from the website. http://transportation.ky.gov/Aviation/Pages/Zoning-Commission.aspx

        - On this permit just fill out one location and add in comments that there are multiple locations.  You will need to attach the spreadsheet to your submittal

        - Send permit to AirportZoning@ky.gov to process and they will send to the KAZC commission for review.

        - There may be follow-up after the structure is installed.

        - Input all locations in to the FAA Notice criteria tool, https://oeaaa.faa.gov/oeaaa/external/gisTools/gisAction.jsp?action=showNoNoticeRequiredToolForm

        - Add column to spreadsheet to note if the tool requires a FAA permit.

        - If the tool requires permit, you will need to get a login ID and fill out a permit for each location.  Make sure that you make KYTC the sponsor and add Jessica Goodwin as the contact for the Sponsor.

        - You may be required by the FAA to update or modify your information that has been turned in on the permit.

        - FAA also will require you to turn in a PART 2 submittal after the structure is installed.  If the structure is moved in the field, a new permit shall be submitted to the FAA.  You will need to terminate the existing location permit.